### 1. 1-bit NOT Gate

Design (Using Continuous Assignment and always block):

// Code your design here

module not_gate_cont (output Y, input A);

assign Y = ~A;

endmodule

module not_gate_always (output reg Y, input A);

always @(*) begin

Y = ~A;

end

endmodule

Testbench:

module not_gate_tb;

reg A; // Input

wire Y; // Output

// Instantiate the NOT gate (Continuous or Always)

not_gate_cont uut (Y, A);  // Change to `not_gate_always` if needed

initial begin

// Set up the dump file and variables for waveform

$dumpfile("dump.vcd");   // Specify the name of the VCD file

$dumpvars(0, not_gate_tb); // Dump all variables in this module

// Apply test cases

A = 1'b0;

#10 A = 1'b1;

#10 $finish;  // End the simulation

end

initial begin

// Monitor the results

$monitor("At time %t: A=%b | Y=%b", $time, A, Y);

end

endmodule

### 2. 2-bit AND Gate

Design (Using Always Block and continuous assignment):

module and_gate_cont (output [1:0] Y, input [1:0] A, B);

assign Y = A & B;

endmodule

module and_gate_always (output reg [1:0] Y, input [1:0] A, B);

always @(*) begin

Y = A & B;

end

endmoduleTestbench:

module and_gate_tb;

reg [1:0] A, B; // Inputs

wire [1:0] Y; // Output

// Instantiate the AND gate (Continuous or Always)

and_gate_cont uut (Y, A, B);  // Change to `and_gate_always` if needed

initial begin

// Set up the dump file and variables for waveform

$dumpfile("and_gate.vcd");

$dumpvars(0, and_gate_tb);

// Apply test cases

A = 2'b00; B = 2'b00;

#10 A = 2'b01; B = 2'b01;

#10 A = 2'b10; B = 2'b10;

#10 A = 2'b11; B = 2'b11;

#10 $finish;

end

initial begin

// Monitor the results

$monitor("At time %t: A=%b, B=%b | Y=%b", $time, A, B, Y);

end

endmodule

### 3. 2-bit OR Gate

Design (Using Continuous Assignment and always block):

module or_gate_cont (output [1:0] Y, input [1:0] A, B);

assign Y = A | B;

endmodule

module or_gate_always (output reg [1:0] Y, input [1:0] A, B);

always @(*) begin

Y = A | B;

end

endmodule

Testbench:

module or_gate_tb;

reg [1:0] A, B; // Inputs

wire [1:0] Y; // Output

// Instantiate the OR gate (Continuous or Always)

or_gate_cont uut (Y, A, B);  // Change to `or_gate_always` if needed

initial begin

// Set up the dump file and variables for waveform

$dumpfile("or_gate.vcd");

$dumpvars(0, or_gate_tb);

// Apply test cases

A = 2'b00; B = 2'b00;

#10 A = 2'b01; B = 2'b01;

#10 A = 2'b10; B = 2'b10;

#10 A = 2'b11; B = 2'b11;

#10 $finish;

end

initial begin

// Monitor the results

$monitor("At time %t: A=%b, B=%b | Y=%b", $time, A, B, Y);

end

endmodule

### 4. 2-bit NAND Gate

Design (Using Always Block and continuous block):

module nand_gate_cont (output [1:0] Y, input [1:0] A, B);

assign Y = ~(A & B);

endmodule

module nand_gate_always (output reg [1:0] Y, input [1:0] A, B);

always @(*) begin

Y = ~(A & B);

end

endmoduleTestbench:

module nand_gate_tb;

reg [1:0] A, B; // Inputs

wire [1:0] Y; // Output

// Instantiate the NAND gate (Continuous or Always)

nand_gate_cont uut (Y, A, B);  // Change to `nand_gate_always` if needed

initial begin

// Set up the dump file and variables for waveform

$dumpfile("nand_gate.vcd");

$dumpvars(0, nand_gate_tb);

// Apply test cases

A = 2'b00; B = 2'b00;

#10 A = 2'b01; B = 2'b01;

#10 A = 2'b10; B = 2'b10;

#10 A = 2'b11; B = 2'b11;

#10 $finish;

end

initial begin

// Monitor the results

$monitor("At time %t: A=%b, B=%b | Y=%b", $time, A, B, Y);

end

endmodule

### 5. 2-bit NOR Gate

Design (Using Continuous Assignment and always block):

module nor_gate_cont (output [1:0] Y, input [1:0] A, B);

assign Y = ~(A | B);

endmodule

module nor_gate_always (output reg [1:0] Y, input [1:0] A, B);

always @(*) begin

Y = ~(A | B);

end

endmodule

Testbench:

module nor_gate_tb;

reg [1:0] A, B; // Inputs

wire [1:0] Y; // Output

// Instantiate the NOR gate (Continuous or Always)

nor_gate_cont uut (Y, A, B);  // Change to `nor_gate_always` if needed

initial begin

// Set up the dump file and variables for waveform

$dumpfile("nor_gate.vcd");

$dumpvars(0, nor_gate_tb);

// Apply test cases

A = 2'b00; B = 2'b00;

#10 A = 2'b01; B = 2'b01;

#10 A = 2'b10; B = 2'b10;

#10 A = 2'b11; B = 2'b11;

#10 $finish;

end

initial begin

// Monitor the results

$monitor("At time %t: A=%b, B=%b | Y=%b", $time, A, B, Y);

end

endmodule

### 6. 2-bit XOR Gate

Design (Using Always Block and continuous block):

module xor_gate_cont(output [1:0] Y, input [1:0] A, input [1:0] B);

assign Y = A ^ B;  // Bitwise XOR operation

endmodule

module xor_gate_always(output reg [1:0] Y, input [1:0] A, input [1:0] B);

always @(*) begin

Y = A ^ B;  // Bitwise XOR operation

end

endmodule

Testbench:

module xor_gate_tb;

reg [1:0] A, B; // Inputs

wire [1:0] Y;   // Output

// Instantiate the XOR gate (Continuous or Always)

xor_gate_cont uut (Y, A, B);  // Use the correct XOR gate implementation

initial begin

// Set up the dump file and variables for waveform

$dumpfile("xor_gate.vcd");

$dumpvars(0, xor_gate_tb);

// Apply test cases

A = 2'b00; B = 2'b00;  // Expect Y = 00

#10 A = 2'b01; B = 2'b00;  // Expect Y = 01

#10 A = 2'b10; B = 2'b00;  // Expect Y = 10

#10 A = 2'b01; B = 2'b11;  // Expect Y = 10

#10 A = 2'b11; B = 2'b10;  // Expect Y = 01

#10 A = 2'b11; B = 2'b11;  // Expect Y = 00

#10 $finish;

end

initial begin

// Monitor the results

$monitor("At time %t: A=%b, B=%b | Y=%b", $time, A, B, Y);

end

endmodule

### 7. 2-bit XNOR Gate

### Design (Using Continuous Assignment and always block):

module xnor_gate_cont (output [1:0] Y, input [1:0] A, B);

assign Y = ~(A ^ B);

endmodule

module xnor_gate_always (output reg [1:0] Y, input [1:0] A, B);

always @(*) begin

Y = ~(A ^ B);

end

endmodule

Testbench:

module xnor_gate_tb;

reg [1:0] A, B; // Inputs

wire [1:0] Y; // Output

// Instantiate the XNOR gate (Continuous or Always)

xnor_gate_cont uut (Y, A, B);  // Change to `xnor_gate_always` if needed

initial begin

// Set up the dump file and variables for waveform

$dumpfile("xnor_gate.vcd");

$dumpvars(0, xnor_gate_tb);

// Apply test cases

A = 2'b00; B = 2'b00;

#10 A = 2'b01; B = 2'b01;

#10 A = 2'b10; B = 2'b10;

#10 A = 2'b11; B = 2'b11;

#10 $finish;

end

initial begin

// Monitor the results

$monitor("At time %t: A=%b, B=%b | Y=%b", $time, A, B, Y);

end

endmodule